# Requirements

- Protocol: Start with SNMPv1 (fast, tiny)

- Function: Support only GET for one OID

- Value: Unique ID That  SNMP server will return when Macmon sends a GET request for your OID.

- Decision (Macmon side): If token matches the taught value → stay on Wi-Fi; else isolate.

- Constraints: Extremely small footprint, low memory/CPU, no external heavy libs.

Small footprint:

- Use minimal memory (tiny buffers, no heavy structures).

- Keep CPU load low (avoid complex loops or encryption).

- Ensure binary size is small for limited storage.

No heavy external libraries:

- Don’t use full SNMP stacks or OpenSSL.

- Implement only what’s needed:

- SNMPv1 GET handler for one OID.

- Simple BER parsing and response builder.

Practical approach:

- Write a custom lightweight SNMP responder.

- Use built-in Winsock for UDP networking.

- Use basic C functions for encoding/decoding

- Generate token with a tiny hash (FNV-1a or CRC32) instead of heavy crypto.

- OS: use Winsock (UDP/161).

# Architecture

(how it fits our current main.c)

We have already:

- parse parameters → InitData

- start MyApp (&InitData)

- wait on ghExitEvent or keyboard to exit

- call _MyApp (&InitData) to clean up

We’ll add a light SNMP agent thread:

main.exe

└─ MyApp (&InitData)

├─ SnmpLight_Start(&InitData, ghExitEvent)              <-- NEW

│            └─ thread: bind UDP/161, handle SNMP GET for one OID

└─ [...existing logic ...]

└─ on exit: └─ SnmpLight_Stop ()

The SNMP thread listens on UDP port 161 (SNMP), decodes GET for the one OID, and sends a GetResponse with your token.

It cleanly exits when ghExitEvent is signalled (or when you call Main_AppExit()).

# Architecture diagram

# Working

- Start the Autoca Program (main.exe)

  - The controller starts the main program.

  - It reads configuration parameters (like car number, IP, etc.) into InitData.

- Run the Main Application (MyApp(&InitData))

  - This is your existing logic that controls the baggage handling car.

  - New addition: Start the SNMP agent by calling SnmpLight_Start(&InitData, ghExitEvent).

- SNMP Agent Thread

  - A small background thread is created.

  - It opens UDP port 161 (standard SNMP port).

  - It waits for SNMP GET requests from Macmon.

  - When Macmon asks for the OID, the thread responds with the unique token (your car’s ID).

- Exit Handling

  - When the system shuts down (or ghExitEvent is triggered), the SNMP thread stops cleanly using SnmpLight_Stop().